# Basic Troubleshooting — Battery / Power Faults

Document Type: procedure_guide | Category: basic_troubleshooting

Standard pre-check procedure for battery and power errors on Desoutter cordless tools (EPBC, EABC, EABS, EIBS series).

## Related Error Codes

E029 (Battery empty)

E031 (Battery error)

E045 (Abnormal battery voltage)

E047 (Battery too low)

E048 (Battery type not allowed)

E230 (DC bus high)

E231 (DC bus too low)

## Pre-Check Procedure

Confirm the battery pack is fully seated in the tool grip — click should be heard/felt when locked.

Place the battery on a calibrated eDOCK charger and verify the charge LED sequence; a flashing red LED indicates a faulty cell.

Check battery firmware version in S-Center under Battery Info — update if the version is below the minimum required for the tool series.

Measure battery terminal voltage with a multimeter; nominal range is 20–21 V (fully charged). Below 18 V indicates deep discharge — recondition or replace.

For E048 (Battery type not allowed), confirm the battery part number matches the tool compatibility table in the product datasheet.

Inspect battery contacts on both the tool and battery pack for oxidation; clean with isopropyl alcohol and a contact brush.

For E230/E231 (DC bus high/low), check the charger output voltage with a multimeter — replace the charger if outside 21–25 V range.

If the battery fails three consecutive charge cycles, mark it as defective and initiate battery replacement through the service management system.

Note: These steps must be performed before escalating to component-level repair. Document findings in the service request.