# Basic Troubleshooting — Connectivity / WiFi Faults

Document Type: procedure_guide | Category: basic_troubleshooting

Standard pre-check procedure for network and communication errors on Desoutter wireless tools and controllers.

## Related Error Codes

E019 (Tool communication error)

E213 (Drive connection lost)

E232 (Error ID fieldbus)

E233 (CVINET FIFO full)

E236 (ToolsNet FIFO full)

E941 (E-Lit WiFi: demo mode expired)

E942 (E-Lit WiFi: not authorized)

## Pre-Check Procedure

Verify the WiFi access point is powered and broadcasting on 2.4 GHz — Desoutter wireless tools do not support 5 GHz networks.

In S-Center, navigate to Tool Network Settings and confirm the SSID, security type (WPA2), and IP address are correctly configured.

Check for IP address conflicts: the tool IP must be unique on the subnet — scan the network with a network tool or check the DHCP lease table on the router.

Confirm the tool is within range of the access point (recommended: within 10 m, no thick concrete walls between tool and AP).

For E941/E942, open S-Center → License Manager and verify the E-Lit WiFi license is active and not expired; renew the license key if needed.

For E232 (Fieldbus error), check the fieldbus cable connection at the Connect Unit rear panel — reseat the connector and verify termination resistor is installed.

For FIFO full errors (E233/E236), check ToolsNet/CVINet server connectivity from the Connect Unit using ping; restart the network service if unresponsive.

Power-cycle the tool and Connect Unit in sequence: Connect Unit first, then the tool — allow 30 seconds between power cycles.

If the tool connects intermittently, update the tool firmware via S-Center to the latest stable version for the product series.

Note: These steps must be performed before escalating to component-level repair. Document findings in the service request.