# Basic Troubleshooting — Drive / Electronics Faults

Document Type: procedure_guide | Category: basic_troubleshooting

Standard pre-check procedure for drive and power electronics errors on Desoutter electric tools and controllers.

## Related Error Codes

E217 (Drive disabled)

E218 (Drive power failure)

E221 (Drive check error)

E223 (Drive init error)

E228 (Drive error)

## Pre-Check Procedure

Check the main power supply to the Connect Unit: verify input voltage is within the rated range (100–240 V AC, 50/60 Hz).

Inspect the power cable and connector between the Connect Unit and the tool for damage, bent pins, or loose contact.

In S-Center, navigate to Drive Status and check the drive enable signal — confirm the emergency stop circuit is not active.

For E217 (Drive disabled), check the I/O configuration in S-Center: verify that the Drive Enable input is wired and active at the PLC/fieldbus level.

For E218 (Drive power failure), measure the DC bus voltage at the drive module test points (nominal 300–340 V DC for 230 V systems); out-of-range voltage requires capacitor inspection.

Run the drive self-test in S-Center → Drive Diagnostics; a failed phase indicates a faulty inverter module.

Check the cooling fan on the Connect Unit for obstruction — overtemperature can trigger drive shutdown; clean the fan filter if blocked.

If drive errors persist after power cycle and self-test, replace the drive module and register the replacement in the service management system.

Note: These steps must be performed before escalating to component-level repair. Document findings in the service request.