# Basic Troubleshooting — Memory / EEPROM Faults

Document Type: procedure_guide | Category: basic_troubleshooting

Standard pre-check procedure for memory and EEPROM errors on Desoutter electric tools.

## Related Error Codes

E012 (Tool EEPROM error)

E034 (Tool memory error)

E035 (Tool memory locked)

## Pre-Check Procedure

Perform a controlled power cycle: remove the tool from the Connect Unit or eDOCK, wait 10 seconds, and reconnect.

In S-Center, navigate to Tool Diagnostics → Memory Test and run the built-in EEPROM self-test; note the pass/fail result.

Check S-Center for a pending firmware update — a version mismatch can cause memory lock (E035); apply the update if available.

Attempt a parameter restore: in S-Center, upload the last known-good configuration backup to the tool.

If E035 (memory locked) persists after firmware update, perform a factory reset via S-Center → Tool Reset → Full Reset; this will erase all Psets — back up first.

If EEPROM errors remain after reset, the tool controller board requires replacement — document the fault and serial number for the service report.

Note: These steps must be performed before escalating to component-level repair. Document findings in the service request.