# Basic Troubleshooting — Motor / Mechanical Faults

Document Type: procedure_guide | Category: basic_troubleshooting

Standard pre-check procedure for motor and mechanical errors on Desoutter electric tools (CVI3, CONNECT, ExBC series).

## Related Error Codes

E006 (Rotor locked)

E007 (Motor too hot)

E008 (Tool angle fault)

E014 (Torque power default)

E018 (Torque out of range)

E227 (Motor stalled)

## Pre-Check Procedure

Verify that the tool spindle rotates freely by hand — if resistance is felt, inspect for foreign material or seized bearing.

Check tool surface temperature: allow the tool to cool down for at least 5 minutes if motor overheat (E007) is reported.

Confirm the tool is correctly seated and locked in the eDOCK or Connect Unit before running a cycle.

In S-Center, navigate to Tool Configuration and verify that the Angle Limit and Torque Limit parameters match the active Pset.

Check the torque transducer cable connector at the tool body for corrosion or loose contact; reseat if necessary.

Run a free-run (no fastener) cycle and observe the angle/torque trace in S-Center — abnormal spikes indicate mechanical obstruction.

Inspect the bit holder and socket for wear or incorrect size that could cause premature stall.

If E227 (motor stalled) persists after clearing the obstruction, replace the motor module and log the serial number in the service report.

Note: These steps must be performed before escalating to component-level repair. Document findings in the service request.