# Basic Troubleshooting — License / Software Faults

Document Type: procedure_guide | Category: basic_troubleshooting

Standard pre-check procedure for license and software errors on Desoutter CVI3 and CONNECT controllers.

## Related Error Codes

E704 (Missing UV)

E705 (Missing demo UV)

E711 (Tightening Unit: demo expired)

E712 (Tightening Unit not active)

E717–E724 (Pset/AP license limits)

E890 (Device software error)

E915 (Inconsistent version)

## Pre-Check Procedure

In S-Center, navigate to License Manager and verify all required feature licenses are present and active (Tightening Unit, Pset count, AP count).

Check license expiry dates — demo licenses (E711, E729, E941) expire; contact Desoutter sales to obtain a permanent activation key.

For E915 (Inconsistent version), compare the firmware version of the tool, Connect Unit, and S-Center — all must be compatible; refer to the Software Compatibility Matrix.

Download the latest compatible firmware package from the Desoutter portal and apply the update via S-Center → Firmware Update in the correct order: Connect Unit first, then tool.

For E890 (Device software error), export the event log from S-Center → Diagnostics → Event Log and check for the root cause error preceding E890.

If a license key is lost or corrupted, retrieve the controller serial number from S-Center → System Info and contact Desoutter support to reissue the license.

After any firmware update, perform a controlled restart of the Connect Unit and re-verify all Psets and AP configurations are intact.

Note: These steps must be performed before escalating to component-level repair. Document findings in the service request.